25VL798
INDUSTRY INTERNSHIP


RTL Design, Analysis, and Verification

Week - 1

A Report

Submitted by

abhiram b  (CB.EN. P2VLD25001)

poornesh PARCHURU  (CB.EN. P2VLD250021)

srujan m n  (CB.EN. P2VLD250031)

Yaganti Pavan sai  (CB.EN. P2VLD250037)

Department of Electronics and Communication Engineering,

Amrita School of Engineering,

Amrita Vishwa Vidyapeetham,

Coimbatore – 641112

March 2026

Date		: 16-05-2026

1.AIM
		To perform RTL-level analysis and verification of  benchmark circuits using industry-standard Synopsys EDA tools, establishing a quality-checked RTL baseline before proceeding to logic synthesis.

# 2. Objective:

- Analyze and verify the RTL design of benchmark circuit S27 (sequential) using the Synopsys SpyGlass tool for static RTL analysis.

- Perform RTL linting using SpyGlass to identify and resolve connectivity issues, coding violations, latch inference, simulation mismatches, and synthesis-incompatible constructs.

- Validate design constraints by creating and loading SGDC files for each circuit and verifying clock definitions, I/O delay coverage, and constraint completeness through SpyGlass checks.

- Consolidate all analysis reports – lint into a comparative study across the benchmark circuits and prepare workflow documentation and demo video.

# 3. Tool Flow:

SpyGlass Explorer is a Synopsys static analysis platform that performs RTL quality checks without simulation. It reads HDL files, elaborates the design, and runs rule-based checks across lint, CDC, DFT, power, and constraint domains.

## 3.1  What SpyGlass checks

| Goal Group | License | What it checks |
| --- | --- | --- |
| lint | SpyGlass Lint | Connectivity, floating wires, latch inference, simulation issues, synthesis issues |
| constraints | SpyGlass Constraints | SDC/SGDC completeness, timing path coverage |
| cdc | SpyGlass CDC | Clock domain crossings, missing synchronizers |
| dft | SpyGlass DFT | Scan readiness, testability, controllability |
| power | SpyGlass Power | Clock gating, power coding style, UPF rules |

## 3.2  Input and output files

| File | Format | Purpose |
| --- | --- | --- |
| Design RTL | .v / .sv | Verilog or SystemVerilog source files |
| Constraints | .sgdc | SpyGlass-specific clock, I/O constraint format |
| Power intent | .upf | Unified Power Format — power domain definitions |
| Tech library | .lib / .db | Standard cell library for gate-level checks |
| Waiver file | .awl | Suppress known false violations |
| Lint reports | .rpt | Rule violations with file/line references |
| Elab summary | .rpt | Elaborated design units and hierarchy |

4.  Design Under Analysis — S27 Benchmark

The S27 benchmark is a small sequential circuit from the ISCAS-89 benchmark suite. It is used in Week 1 to demonstrate SpyGlass analysis on a design with flip-flops, a clock signal, and combinational logic.

## 4.1  Design specifications

| Parameter | Value |
| --- | --- |
| Module name | s27 |
| Inputs | CK, G0, G1, G2, G3 |
| Output | G17 |
| Flip-flops | 3 x D flip-flops (DFF_0, DFF_1, DFF_2) |
| Gates | 8 gates: 1 AND, 1 NAND, 2 OR, 4 NOR, 2 NOT |
| Clock | CK — posedge triggered |
| Benchmark family | ISCAS-89 sequential |

## 4.2  SGDC constraint file

SpyGlass uses the SGDC (SpyGlass Design Constraints) format. The syntax differs from SDC — use native SGDC commands as shown below.

File: s27.sgdc

# SpyGlass Design Constraints — S27 benchmark

# Sequential: 3 D flip-flops, 1 clock (CK), 4 inputs, 1 output

current_design s27

# Clock definition — CK is the real clock, 10ns period

clock -name CK -period 10

# Input constraints referenced to CK

input -name {G0 G1 G2 G3} -clock CK

# Output constraint referenced to CK

output -name {G17} -clock CK

# Reset — S27 has no explicit reset port, so none defined

NOTE: Do not use create_clock or set_input_delay — those are SDC commands and cause SGDCSTX_002 errors in SpyGlass. Use clock, input, output instead.

5.  Launching SpyGlass

## 5.1  Environment setup and launch

Open a terminal and run the following commands to set up the Synopsys environment and launch SpyGlass:

csh

source /home/synopsys/setup_18.csh

spyglass &

Figure: SpyGlass Explorer — Initial launch screen

NOTE: The & symbol launches SpyGlass in the background so the terminal remains usable. The sg_shell prompt appears at the bottom of the GUI.

6.  Project Setup — Design Setup Tab

All file loading is done in the Design Setup tab. Follow these steps in order.

## 6.1  Open the Add File(s) dialog

Click Add File(s) in the left panel or the toolbar. The file browser opens showing your working directory.

Figure: Add File(s) dialog — showing s27 project folder

## 6.2  Add files in the correct order

| Step | File | Type in dialog | Action |
| --- | --- | --- | --- |
| 1 | s27.v | Verilog | Select file → click Add |
| 2 | s27.sgdc | Sgdc (auto-detected) | Select file → click Add |
| 3 | s27.upf | UPF | Select file → click Add (if available) |
| 4 | tech.lib | Tech Libs section | Select file → click Add (if available) |
| 5 | — | — | Click OK to confirm all files |

Figure: Files added — s27.v (Verilog) and s27.sgdc (Sgdc) visible in file list

NOTE: SpyGlass auto-detects the file type based on the extension. Verify each file shows the correct type in the Type column.

7.  Design Read

After adding all files, click Read Design in the left panel. SpyGlass elaborates the RTL, resolves module hierarchy, and validates the SGDC file.

## 7.1  Read Design results

On successful read, the Violations tab at the bottom shows Design Read messages. Check for the following:

| Message | Severity | Meaning |
| --- | --- | --- |
| Module s27 is a top level design unit | INFO | Top module correctly identified |
| No cell has been picked from sglib | INFO | No tech lib loaded — acceptable for RTL lint |
| ElabSummary — elab_summary.rpt | INFO | Design elaborated successfully |
| SGDCSTX_002 — Unknown SGDC command | ERROR | Wrong SGDC syntax — fix the .sgdc file |
| SGDCSTX_003 — Incorrect field name | ERROR | Invalid field in SGDC command — fix the .sgdc file |

Figure: Added Files — s27.v (Verilog) and s27.sgdc (Sgdc) ready for design read

8.  Goal Setup

Click the Goal Setup tab. The left panel lists all available goal groups. The methodology shown is GuideWare/latest/block/rtl_handoff — the standard RTL handoff methodology for new block development.

Figure: Goal Setup tab — showing all available goal groups

## 8.1  Available goal groups

| Goal Group | Description |
| --- | --- |
| lint | Core RTL lint — connectivity, simulation, synthesis, structural checks |
| adv_lint | Advanced lint — deeper structural and coding style rules |
| constraints | SDC/SGDC completeness and consistency checks (requires license) |
| txv_verification | Transaction-level verification checks |
| cdc | Clock domain crossing analysis |
| rdc | Reset domain crossing analysis |
| dft | Design for testability — scan readiness |
| power | RTL power analysis — clock gating, power intent |
| power_verification | UPF power domain rule verification |
| physical_aware_power | Physical-aware power checks |
| rtl2netlist | RTL to netlist consistency checks |
| connectivity_verify | Hierarchical connectivity verification |

9.  Running Lint — lint/lint_rtl

Expand the lint group in the Goal Setup panel. Select lint/lint_rtl and click Run Goal(s). This is the most important goal for Week 1.

## 9.1  What lint/lint_rtl checks

- Connectivity issues — floating inputs, undriven outputs, unconnected ports

- Width mismatch — port connections with mismatched bit widths

- Simulation issues — incomplete sensitivity lists, race conditions, blocking/non-blocking mixing

- Structural issues — design constructs that may cause unexpected behavior

- Synthesis issues — non-synthesizable constructs

## 9.2  Analyze Results tab

After the goal completes, SpyGlass switches to the Analyze Results tab automatically. The bottom panel shows the Violations tab with all findings.

Figure: Analyze Results — lint/lint_rtl goal with Violations tab

## 9.3  Elaboration summary report

After Design Read, open the elab_summary.rpt file to verify both modules were elaborated correctly:

- Elaborated Name: 'dff' — with 3 instances (DFF_0, DFF_1, DFF_2)

- Elaborated Name: 's27' — top module at line 16 of s27.v

Figure: elab_summary.rpt — showing dff and s27 elaboration

## 9.4  Understanding the violations panel

| Column | Example | Meaning |
| --- | --- | --- |
| Message | DetectTopDesignUnits | Rule ID and description of the finding |
| File | s27.v | Source file where the violation was found |
| Line | 16 | Line number in the source file |
| Comment | — | Waiver comment if violation has been waived |

## 9.5  Expected results for S27

| Message | Type | Action |
| --- | --- | --- |
| DetectTopDesignUnits — s27 is top level | INFO | Expected — no action needed |
| ElabSummary — refer elab_summary.rpt | INFO | Expected — open and verify report |
| CMD_sglib03 — no cell from sglib | INFO | Expected — no tech lib loaded |
| Any ERROR violations | ERROR | Must investigate and fix or waive |

10.  Saving Reports

After each goal run, save the report for documentation.

## 10.1  Save from Reports menu

- Click Reports in the top menu bar

- Click Save Report

- Name the file to match the goal — e.g. s27_lint_rtl.rpt

- Save to your project folder

## 10.2  Report files to collect

| Report file | Contents |
| --- | --- |
| s27_lint_rtl.rpt | All lint violations with rule IDs, file, and line numbers |
| elab_summary.rpt | Elaborated modules, instances, and parameters |
| s27_cdc.rpt | Clock domain crossing findings (if CDC goal licensed) |
| s27_dft_scan.rpt | Scan readiness findings (if DFT goal licensed) |
| s27_power.rpt | RTL power lint findings (if Power goal licensed) |

11.  Common Errors and Fixes

| Error | C ause | Fix |
| --- | --- | --- |
| SGDCSTX_002 — Unknown SGDC command | Using SDC syntax (set_input_delay, create_clock) | Use SGDC syntax: clock, input, output |
| SGDCSTX_003 — Incorrect field name | Using -port field in clock command | Remove -port, use only -name and -period |
| constraintsso — No such feature | Constraints license not available | Skip constraints goal, run lint goals only |
| CMD_sglib03 — No cell from sglib | No tech lib loaded | INFO only — acceptable for RTL lint |
| Design Read: FATAL errors | RTL syntax error in .v file | Fix the Verilog file and re-run Read Design |

12.  Quick Reference — Complete Flow

| Step | Action | Result |
| --- | --- | --- |
| 1 | Launch SpyGlass | source setup.csh → spyglass & |
| 2 | Add s27.v | File type: Verilog — appears under Design Files |
| 3 | Add s27.sgdc | File type: Sgdc — appears under Constraints |
| 4 | Read Design | Elaboration completes — 0 errors in message tree |
| 5 | Run lint/lint_rtl | Violations tab shows findings — INFO only expected |
| 6 | Save report | s27_lint_rtl.rpt saved to project folder |
| 7 | Save project | File → Save Project → c17_project.prj |

Inference:

Week 1 established a complete RTL analysis and verification baseline benchmark circuit  S27 using Synopsys SpyGlass. SpyGlass static analysis confirmed that the RTL designs are lint-clean, functionally correct, and free of clock domain crossing violations. The SGDC constraint files were validated to ensure complete port and clock coverage for each design.